Eyal Espresso Server Simulator Detailed Design

Document Status: Working Design Baseline

Project Version Reference: 0.1.15

Reviewed on: 12-Mar-26 21:05:26

Owner: Eyal / Codex

1. System Context and Planned Split Architecture

Purpose: this document defines the new simulator architecture in which the operator-facing simulator application runs on the PC, but low-level Wi-Fi/TCP transport is delegated to an ESP32-C3-SuperMini attached over USB as a COM port.

Deployment Topology:

PC simulator application -> USB serial COM link -> ESP32-C3-SuperMini transport controller -> Wi-Fi TCP link -> ESP32-S3 client

Architecture Decision: the PC remains the simulator brain, operator UI, scenario engine, and supervisory control surface. The ESP32-C3 remains the authoritative low-level transport controller, including live-integer ownership, keepalive/watchdog enforcement, and CRC validation.

Why This Split Is Preferred: it preserves debuggability and operator visibility on the PC while isolating timing-sensitive low-level transport control in a dedicated microcontroller.

Figure 1. Simulator-side split architecture showing the PC host, USB bridge, ESP32-C3 transport controller, and client link.

2. Responsibility Allocation

2.1 PC Simulator Application Responsibilities

- operator UI and workflow controls

- simulator state machine and scenario logic

- log aggregation and packet trace presentation

- framing and interpretation of low-level commands sent to the ESP32-C3

- mirrored state presentation, logging, and operator controls

- COM-port ownership, forced COM release, and USB session lifetime control

- mirrored configuration validation before INITIALIZE is sent

- reset and initialize policy decisions after failures reported by the bridge

- sequential keepalive-failure counting and ConnectionFault latching into wait_for_com_reset

2.2 ESP32-C3-SuperMini Responsibilities

- own the USB serial session exposed to the PC

- parse, validate, and emit low-level frames

- enforce CRC validation before any payload is treated as valid

- own the Wi-Fi join process and TCP socket establishment toward the client

- maintain the low-level state machine: reset, initialize, connect, keepalive, and bridge fault reporting

- own ServerLiveInteger and ClientLiveInteger during the active session

- zero both counters every time connect is entered

- enforce the 100 mSec keep-alive and watchdog behavior

2.3 Client Responsibilities

- participate in the mirrored low-level transport state machine

- accept validated payloads only after CRC, sequencing, and liveness checks pass

- expose higher-level machine protocol behavior above the low-level transport layer

2.4 Current Implemented Split: PC Simulator Runtime

Current Main Files:

- server/app.py = FastAPI entry point that serves /health, /api/app-info, the API router, and the browser UI shell

- server/api/routes.py = operator and test API endpoints that call the low-level runtime actions

- server/sim/link_state_machine.py = current host-side mirror of authoritative bridge state

- server/transport/serial_link.py = one-owner serial manager for the COM endpoint

- server/transport/frame_codec.py = framed packet encoder/decoder with CRC16

Implemented Low-Level Server States:

- reset

- initialize

- connect

- keepalive

- wait_for_com_reset

Main Server Runtime Functions:

- LinkRuntime.configure_port() = update the active serial port target

- LinkRuntime.configure_transport() = update mirrored serial, Wi-Fi, TCP, and watchdog configuration

- LinkRuntime.open_transport() = open and own the configured COM endpoint

- LinkRuntime.close_transport() = release the configured COM endpoint and clear transport-ready flags

- LinkRuntime.force_release_transport() = hard COM-port release helper for likely external holders

- LinkRuntime.toggle_wifi_enabled() = operator test hook for bridge SoftAP availability

- LinkRuntime.reset() = clear mirrored host state, send RESET, and arm automatic initialize progression

- LinkRuntime.initialize() = validate mirrored configuration, send INITIALIZE, and schedule the automatic CONNECT stage

- LinkRuntime.send_keepalive() = monitor-only API that reports bridge-owned keepalive state

- LinkRuntime.send_data() = transmit DATA only after the bridge has reported keepalive-ready state

- LinkRuntime.get_snapshot() = return the current operator-visible runtime snapshot

- SerialLinkManager.open_port() / close_port() / send_frame() = the only code paths allowed to touch the serial object

Current UI Integration:

- server/ui/index.html = browser operator dashboard that drives Screen 1, Screen 2, and Screen 3 through the API routes

2.5 Current Implemented Split: ESP32-C3 Transport Controller Firmware

Current Main Files:

- firmware/esp32c3_bridge/main/bridge_main.c = current ESP32-C3 transport-controller baseline

Implemented Bridge States:

- BRIDGE_STATE_RESET

- BRIDGE_STATE_INITIALIZE

- BRIDGE_STATE_CONNECT

Implementation Note: the current firmware source still carries legacy internal bridge enums such as BRIDGE_STATE_DISCONNECT and BRIDGE_STATE_ERROR, but the active host-visible transport contract is reset -> initialize -> connect -> keepalive with wait_for_com_reset handled by the Python supervisor.

Implemented Bridge Message Types:

- BRIDGE_MESSAGE_RESET

- BRIDGE_MESSAGE_INITIALIZE

- BRIDGE_MESSAGE_CONNECT

- BRIDGE_MESSAGE_DISCONNECT

- BRIDGE_MESSAGE_KEEPALIVE

- BRIDGE_MESSAGE_ERROR

- BRIDGE_MESSAGE_ACK

- BRIDGE_MESSAGE_DATA

Main Bridge Functions:

- app_main() = USB-Serial/JTAG polling loop that reads frames, services TCP transport, and enforces the bridge watchdog

- bridge_transport_init() = install and configure the USB-Serial/JTAG transport used as the COM endpoint

- bridge_parse_frame() = validate framing, payload length, and CRC before accepting a request

- bridge_handle_frame() = move the bridge state machine and emit the matching ACK or KEEPALIVE response

- bridge_send_frame() = serialize and transmit one framed response back to the PC simulator host

- bridge_service_transport_watchdog() = authoritative keepalive supervision and stale-session teardown on the bridge

- bridge_crc16_ccitt() = shared integrity algorithm used for bridge-side frame validation and emission

Current Bridge Scope Limitation:

- the ESP32-C3 firmware is now the low-level transport authority, but higher-level simulator scenarios and UI orchestration still live on the PC host

3. Low-Level State Machine Definition

The lowest communication layer shall implement the same conceptual state model on all participating sides so failure handling is deterministic.

States:

reset

initialize

connect

keepalive

wait_for_com_reset

State Semantics:

reset = hard reset, counter clear, buffer clear, ESP controller re-initialization, and automatic initialize arming

initialize = load the ESP controller with mirrored Wi-Fi and server constants before automatic connect

connect = wait for the ESP32-C3 bridge to report explicit client-connected or keepalive traffic

keepalive = active low-level supervision owned by the ESP32-C3 bridge after connect success

wait_for_com_reset = host-side latch that waits for explicit Reset Communication after repeated bridge-reported keepalive faults

Primary State Diagram:

reset -> initialize -> connect -> keepalive

keepalive -> connect when the bridge reports transport loss

keepalive -> wait_for_com_reset after 5 sequential bridge-reported keepalive failures

wait_for_com_reset -> reset

Figure 2. Host-visible low-level transport state machine, with bridge-side timing ownership.

Transport Controller Execution Notes:

- reset shall clear stale socket ownership and stale COM-port session assumptions, then arm automatic initialize progression

- initialize shall verify that Wi-Fi credentials, TCP role, target address, and watchdog values are coherent before automatic connect

- connect shall zero both live integers at the bridge before the first keepalive of a new session

- the Python host shall not derive transport watchdog failures from browser snapshot timing

- keepalive shall not forward application data upward until the bridge reports a healthy exchange

3.1 Mirrored Runtime Configuration Defaults

The simulator host shall mirror the same low-level transport defaults used by the client-side communication layer so both repositories describe one coherent interface contract.

Default Fields:

serial_port = COM4

wifi_ssid = EyalSimulatorAP (advertised as a visible SoftAP by the ESP32-C3 bridge)

wifi_password = espresso1234

server_ip = 192.168.4.1

server_port = 3333

wifi_connect_timeout_ms = 10000

tcp_connect_timeout_ms = 3000

keepalive_period_ms = 100

Simulator Rule: the PC host stores and displays this mirrored configuration even though the ESP32-C3 bridge remains the actual owner of the physical Wi-Fi/TCP link.

4. Standard Low-Level Frame Envelope

Recommendation: use one standard low-level frame envelope with multiple packet types. Do not force every exchange into one giant all-data packet.

Reasoning: a stable envelope keeps framing and integrity consistent, while dedicated packet types keep state transitions, watchdog traffic, and application data unambiguous.

Recommended Frame Fields:

SOF

protocol_version

message_type

flags

payload_length

host_live_integer

device_live_integer

sequence

payload

crc

CRC Policy: low-level integrity checking belongs entirely in this envelope layer so the upper-level controller state machine does not need a second checksum mechanism.

CRC Recommendation: use CRC16 or CRC32 instead of a weak additive checksum.

Liveness Recommendation: ServerLiveInteger and ClientLiveInteger are authoritative from the ESP32-C3 bridge/client transport exchange. The Python simulator host mirrors those values and does not invent them.

5. Packet Types

RESET packet: requests hard reset and self-test.

INITIALIZE packet: requests low-level resource preparation using reset-defined parameters.

CONNECT packet: requests active connection establishment or active-session entry.

KEEPALIVE packet: dedicated 100 mSec liveness frame even when no payload changed.

DATA packet: carries application-level simulator or controller payload after the low-level link is healthy.

ERROR packet: reports latched low-level fault information.

ACK packet: acknowledges a command or transport event where a dedicated specialized ACK is sufficient.

6. Flow Diagrams for Each Packet Type

6.1 RESET Packet Flow

PC Host -> RESET(parameters or parameter profile)

ESP32-C3 -> enter reset, clear counters, clear partial frames, run self-test

ESP32-C3 -> RESET_ACK(result, self-test summary)

PC Host runtime -> if reset succeeded then automatically enter initialize

Figure 3. Simulator RESET packet flow from host command into bridge self-test and acknowledgement.

6.2 INITIALIZE Packet Flow

PC Host -> INITIALIZE(serial port, Wi-Fi SSID/password, server IP/port, watchdog settings)

Simulator host -> validate mirrored COM and Wi-Fi/TCP configuration

ESP32-C3 -> validate configuration and prepare low-level resources

ESP32-C3 -> INITIALIZE_ACK(status, validation detail)

PC Host runtime -> if initialize succeeded then automatically enter connect

Figure 4. Simulator INITIALIZE packet flow for Wi-Fi/TCP preparation and validation.

6.3 CONNECT Packet Flow

PC Host -> CONNECT(server endpoint reference)

ESP32-C3 -> zero ServerLiveInteger and ClientLiveInteger and establish Wi-Fi-ready state

ESP32-C3 -> CONNECT_ACK(success or failure detail)

ESP32-C3 -> if a TCP client is already present, immediately mirror client_connected and the first KEEPALIVE

Figure 5. Simulator CONNECT packet flow that enters the active supervised link state.

6.4 KEEPALIVE Packet Flow

Every 100 mSec ESP32-C3 -> KEEPALIVE(ServerLiveInteger, ClientLiveInteger)

ESP32-S3 client -> validate ServerLiveInteger and increment ClientLiveInteger

ESP32-S3 client -> KEEPALIVE(ClientLiveInteger incremented)

ESP32-C3 -> verify the returned ClientLiveInteger and advance ServerLiveInteger

Python simulator host -> mirror the authoritative bridge counters over USB

Figure 6. Simulator KEEPALIVE packet flow with bridge-owned liveness and host-side mirroring.

6.5 DATA Packet Flow

PC Host simulator logic -> DATA(application payload)

ESP32-C3 -> validate frame and CRC

ESP32-C3 -> transmit payload over Wi-Fi TCP to client

Client -> respond with DATA or ACK

ESP32-C3 -> forward validated response to PC host

Figure 7. Simulator DATA packet flow showing bridge-side validation before forwarding.

6.6 ERROR Packet Flow

ESP32-C3 bridge or client low-level layer detects fault

Faulting side -> ERROR(error code, state, counters, transport summary)

Python host -> retry through connect for bridge-reported keepalive loss or latch wait_for_com_reset for blocking COM/runtime faults

Recovery command must be reset once wait_for_com_reset is reached

Figure 8. Simulator ERROR packet flow preserving low-level fault context for explicit recovery.

7. Keep-Alive and Watchdog Policy

Timing Rule: every healthy connected session shall exchange keep-alive traffic every 100 mSec.

Failure Trigger: if the ESP32-C3 bridge does not receive the expected client keepalive progress within the watchdog window, it shall report keepalive_supervision_lost upstream and close the stale TCP session.

Counter Ownership Rule: the bridge zeros both counters when connect begins, then ServerLiveInteger starts on the bridge side and ClientLiveInteger advances only after the client validates the received server value.

Host Visibility Rule: the Python simulator host mirrors bridge-reported counters and faults; it does not derive keepalive loss from UI polling cadence.

Why Not Only a Giant Standard Data Packet: if keep-alive is embedded only in full-state data packets, then transport health becomes coupled to application payload frequency. Dedicated keep-alive frames are more reliable.

8. Failure Modes and Respective State Diagrams

A low-level transport fault shall be detected first by the ESP32-C3 bridge when it relates to the active TCP keepalive session. The Python host mirrors those faults and applies operator recovery policy.

Figure 9. Simulator failure overview showing bridge-owned transport detection and host-side recovery policy.

8.1 CRC Failure

Description: received frame fails CRC validation and must not be forwarded upward.

Diagram:

connect or keepalive -> receive invalid CRC -> reject frame and retry transport progression

Recovery:

automatic retry through connect, then explicit reset after threshold if required

8.2 Bridge Keepalive Watchdog Failure

Description: the ESP32-C3 bridge does not receive the expected client keepalive progression within the watchdog window.

Diagram:

keepalive -> bridge watchdog timeout -> host retry through connect

Recovery:

repeated bridge watchdog failures -> wait_for_com_reset -> reset after operator action

8.3 Device-Side Keepalive Failure

Description: the ESP32-S3 client does not provide the expected ClientLiveInteger progression.

Diagram:

keepalive -> bridge observes missing client forward progress -> bridge reports keepalive_supervision_lost -> host retry through connect

Recovery:

automatic retry until threshold, then wait_for_com_reset -> reset

8.4 USB COM Failure

Description: the PC can no longer exchange low-level frames with the ESP32-C3 bridge.

Diagram:

reset/initialize/connect/keepalive -> COM failure -> wait_for_com_reset

Recovery:

restore COM availability -> reset

8.4A COM Port Not Found

Description: the simulator host cannot open the configured COM port while preparing the bridge session.

Simulator Error Text: COM port not found.

Diagram:

open or initialize -> COM port unavailable -> wait_for_com_reset

Recovery:

restore COM availability -> reset

8.5 Wi-Fi Association Failure

Description: the bridge-side connect path cannot establish the required Wi-Fi/TCP transport preconditions.

Diagram:

initialize or connect -> bridge-side Wi-Fi/TCP failure -> retry through connect

Recovery:

automatic retry or explicit reset after operator action

8.5A Configured AP Offline or Not Visible

Description: the mirrored Wi-Fi configuration indicates an unavailable AP or bridge-side validation reports that the target AP is not visible.

Simulator Error Text: Wi-Fi AP is offline.

Diagram:

initialize -> validate mirrored Wi-Fi config -> AP unavailable -> wait_for_com_reset

Recovery:

correct RF environment or Wi-Fi configuration -> reset -> initialize

8.5B TCP Server Not Found or Not Listening

Description: the bridge-side connect path cannot reach the configured server endpoint as an active listener.

Simulator Error Text: TCP server not found.

Diagram:

connect -> TCP listener unavailable -> retry through connect

Recovery:

restore listener or correct endpoint -> initialize -> connect

8.5C Generic Unknown Transport Failure

Description: a transport stage fails without enough evidence to classify the fault as COM, Wi-Fi AP, or TCP listener specific.

Simulator Error Text: generic unknown failure.

Diagram:

initialize/connect -> stage-specific failure without precise classification -> retry or wait_for_com_reset depending on the failing stage

Recovery:

review stage detail -> reset or initialize

8.6 TCP Session Loss

Description: Wi-Fi may still be up while the TCP socket to the client is gone.

Diagram:

keepalive -> socket loss -> retry through connect

Recovery:

initialize -> connect

8.7 Malformed Packet or Unsupported Version

Description: parser rejects packet structure before payload is considered valid.

Diagram:

initialize/connect/keepalive -> parser reject -> drop frame and retry transport progression

Recovery:

automatic retry or reset after protocol review

8.8 Reset-Only Error Recovery

Description: any latched wait_for_com_reset condition may be cleared only by reset.

Diagram:

wait_for_com_reset -> reset -> initialize -> connect

Recovery:

no direct initialize-from-error path is allowed

9. Simulator Software Module Allocation

Current Runtime Stack: Python, FastAPI, pyserial, uvicorn.

Current PC-Side Code Boundaries:

ServerInterface/frame_codec.py = shared Python reference implementation of the portable transport framing rules

ServerInterface/native/include/server_interface/*.hpp and c_api.h = portable native protocol headers intended for MCU reuse

ServerInterface/native/src/protocol.cpp = native framing and CRC implementation intended for later STM32 reuse through the same C API

server/app.py = FastAPI entry point

server/api/routes.py = API endpoints used by the operator UI

server/sim/controller_state.py = simulator-side state and telemetry model

server/sim/link_state_machine.py = mirrored low-level COM + Wi-Fi/TCP state model and error mapping

server/transport/serial_link.py = logical transport ownership model

server/transport/frame_codec.py = compatibility shim that re-exports the extracted ServerInterface Python reference module

Shared-Interface Rule: transport framing, CRC, packet enums, and later low-level state-machine entry points shall move into ServerInterface first so the PC server and future STM32 firmware can exercise the same interface contract.

Planned Firmware Boundary: the ESP32-C3 firmware should become a separate low-level transport project rather than being hidden inside the PC simulator application.

10. Environment and Local Run Procedure

Local Environment Setup: create a virtual environment with python -m venv .venv.

Dependency Install: .\.venv\Scripts\python.exe -m pip install -r requirements.txt pytest

Normal Run Command: ./scripts/run_simulator.ps1

Development Run Command: ./scripts/run_simulator.ps1 -Reload

Default Access URL: http://127.0.0.1:8000

Repository Version Tree:

- Application Version = 0.1.15 from repository root VERSION

- Firmware Version = 0.1.15 from firmware/esp32c3_bridge/VERSION

Version Tree Rule: application and firmware versions use the same X.Y.Z policy but remain independently tracked artifacts inside one repository version tree.

Runtime Version Baseline:

- Python = 3.13.3

- fastapi = 0.135.1

- uvicorn = 0.41.0

- pyserial = 3.5

Versioning Rule: these runtime versions define the local development baseline for the manual simulator launcher and installation verification flow.

Installation Verification Design:

- the manual launcher shall verify the required local runtime components inside the single visible launcher host before it attempts any simulator startup step

- the required development-baseline runtime shall match the Runtime Version Baseline listed in this section using the repository-local .venv

- if any required component is missing or its version differs from the development baseline, show a popup listing the gaps and stop the launch

- do not continue into backend startup when installation verification fails

Backend Launch Design:

- use a deterministic repo-local launcher script or service wrapper

- prefer the repository virtual environment interpreter before a global Python installation

- before launching uvicorn, inspect any existing listener already bound to the requested simulator port and only replace it when it is positively identified as this simulator backend

- prefer graceful shutdown before forced termination when replacing an existing simulator listener

- capture stdout and stderr to logs or keep them visible in the supervising console

- require a concrete readiness signal such as GET /health before opening the UI

- supervise the process with a stable host if it must outlive the initiating shell

- separate application correctness from editor, sandbox, or task-runner lifetime

Preferred Uvicorn Launch Contract:

- scripts/run_simulator.bat shall open one temporary PowerShell session for the standard-user launch path and close it after backend readiness and the browser prompt complete

- scripts/launch_simulator_ui.ps1 shall own installation verification, backend readiness waiting, and the browser-open prompt inside that same launcher host

- launch scripts/run_simulator.ps1 from the repository root for backend-only startup logic

- let the script resolve .\.venv\Scripts\python.exe when available

- stop a pre-existing process only when the launcher can verify from process metadata and command line that the listener belongs to this simulator backend

- refuse startup rather than kill an unrelated service that happens to own the target port

- attempt normal process termination first, then force-stop only if the old simulator backend does not exit in time

- the manual launcher shall start the backend as a Python process directly rather than opening a second visible PowerShell host

- start uvicorn as server.app:app on 127.0.0.1:8000

- verify readiness with GET /health expecting 200 OK and {"status":"ok"}

- open the browser only after readiness succeeds

- for the manual batch launcher, show a Yes/No prompt after backend readiness asking whether to open the simulator in a fresh browser session with a cache-busting URL

- the startup splash screen shall show the application name, simulator application version, bridge firmware version, and backend build timestamp for 5 seconds

- bypass stale browser state with a cache-busting query string when opening the UI

Operational Note: if a foreground uvicorn launch works but a detached tool-hosted launch dies immediately, classify the problem as process-hosting automation first, not as a backend application defect.

Helper Script Inventory:

- scripts/run_simulator.ps1 = primary repo-local simulator launcher that resolves the repository root, prefers the local .venv interpreter, safely replaces only verified stale simulator listeners on the target port, and starts uvicorn

- scripts/verify_simulator_installation.ps1 = shared manual-launch preflight check that verifies required runtime components and exact development versions

- scripts/launch_simulator_ui.ps1 = single-host manual-launch helper that verifies installation, starts the backend process, waits for /health, and shows a browser-open confirmation message box

- scripts/run_simulator.bat = Windows batch wrapper that opens a temporary PowerShell session for scripts/launch_simulator_ui.ps1 and closes it after the prompt flow completes

- scripts/setup_idf_env.ps1 = prepares the local ESP-IDF shell environment for firmware-side commands

- scripts/idfw.ps1 = PowerShell helper that forwards ESP-IDF commands through the local environment activation flow

- scripts/idfw.cmd = cmd wrapper for the ESP-IDF PowerShell launcher so Windows shells can invoke firmware workflows consistently

- scripts/monitor_capture.ps1 = fallback serial capture helper that reads the target port without reset for bounded log collection

- scripts/play_build_success_sound.ps1 = one-shot celebration sound helper for successful verification cycles

- scripts/play_wait_sound.ps1 = one-shot wait-notification playback helper

- scripts/start_wait_sound.ps1 = starts the repeating wait-sound worker and records its process handle

- scripts/stop_wait_sound.ps1 = stops the repeating wait-sound worker using the recorded handle

- scripts/generate_requirements_docx.ps1 = generates the requirements-and-design .docx artifact from repo-local OpenXML content

- scripts/generate_detailed_design_docx.ps1 = generates this detailed design .docx artifact and embeds the maintained diagram images

- scripts/generate_transport_diagrams.ps1 = renders the maintained transport architecture diagrams used by the design set

11. Installation and Rapair Design

Purpose: this chapter defines how future installer and repair flows shall create, verify, and restore the simulator runtime environment so local setups remain aligned with the development baseline.

Best-Practice Installation Rules:

- treat the repository .venv as disposable and reproducible rather than as a permanent hand-maintained asset

- create the virtual environment from the intended Python version explicitly instead of relying on whatever python happens to be on PATH

- install packages through the repository-local interpreter or its pip entry point so installation always targets the correct .venv

- pin exact dependency versions with == for repeatable installs

- prefer stronger reproducibility with hashes for tightly controlled or release-oriented installer flows

- treat the .venv as non-portable and recreate it when the environment is moved, damaged, or version-drifted

- run verification after install instead of assuming installation success implies runtime correctness

Development Alignment Rules:

- the installer shall compare the local runtime against the documented Runtime Version Baseline

- alignment checks shall include the Python version and the exact package versions required by the simulator

- the launcher or installer shall fail fast when versions drift or required components are missing

- any failure report shall list concrete gaps so the operator knows exactly what is missing or mismatched

Verification Scope for Installer and Repair:

- verify the repository-local Python interpreter exists

- verify the Python version matches the development baseline exactly

- verify each required runtime package is installed

- verify each required runtime package version matches the development baseline exactly

- verify the environment is not carrying stale or partially uninstalled distribution artifacts

- run package-health validation after installation or repair

Preferred Repair Philosophy:

- a damaged or drifted .venv should normally be rebuilt, not patched manually

- the safest repair path is remove -> recreate -> reinstall -> reverify

- surgical package repair may be used only for clearly isolated minor issues, but full rebuild remains the default reliable method

Repair Flow Design:

1. detect a missing component, version mismatch, or broken package artifact

2. present the gap list clearly to the operator

3. offer a Repair action instead of requiring manual reconstruction

4. delete the repository .venv

5. recreate the .venv using the required Python version

6. reinstall the pinned baseline dependencies

7. rerun verification

8. report success or failure with a clear popup summary

Installer Integration Expectation:

- when a future installer is implemented, it shall read the version-baseline and verification requirements from this chapter and use them as the source of truth for setup and repair behavior

Why This Design Is Preferred:

- rebuilding the environment is usually more reliable than trying to patch a partially corrupted venv

- explicit verification makes local runtime drift visible before the simulator launch fails later

- one-step repair keeps the workflow practical for operator test stations and development machines

12. Simulator UI State and Command Feedback

Command Button Rule: simulator command buttons use blue as the default unpressed color.

Pressed/In-Progress Rule: when a command button is pressed it turns gray and looks pressed while the related command or state transition is still in progress.

Completion Rule: after the related state/action finishes, the command button returns to the default blue unpressed state.

Server State Color Rules:

dark blue = inactive / default after reset

blinking green = in progress

red = finished with failure

light green = finished with success

Grouping Rule: server-state indication shall be shown in a dedicated titled group box named Server States so state reporting remains visually separate from operator commands.

Editable Text Rule: all editable text controls in the simulator UI shall use a bright fill with a light visible frame so writable fields are visually distinct from static text boxes.

Logger Rule: the logger remains a separate rolling history surface and is not itself a machine-state indicator.

13. Notes

This design intentionally separates low-level transport supervision from upper-level simulator behavior so watchdog correctness, CRC validation, and failure recovery remain auditable.

The packet and state diagrams above are meant to be reviewed as the source-of-truth baseline before firmware and client-side low-level code are expanded further.